# Challenge / Project

## Final Objective

WHAT TO DO?

Develop a comprehensive understanding of the object tracking using sparse optical flow. Develop a program that tracks an object using Lucas-Kanade method.

HOW SHOULD IT WORK?

Go through the assignment materials and solve the intermediate tasks.

Required knowledge

Introduction to Optical Flow and Sparse Optical Flow

1 hour

1

Manual keypoint selection

30 min

Task 1

Tracking using sparse optical flow

1 hour

Task 2

Activity 3: Sparse optical flow for object tracking

2

## Required knowledge

Required knowledge to be introduced to students

1 HOUR

Optical flow is a computer vision technique used to describe image motion. It is usually applied to a series of images that have a small time step between them - one example is video frames. Optical flow calculates a velocity for points within the images, and provides an estimation of where points could be in the next image sequence.

Optical flow works on two assumptions:

- The pixel intensities of an object do not change between consecutive frames.

- Neighbouring pixels have similar motion.

You may already be familiar with optical flow from the learning module 1 - in this task we will present only the use cases of sparse and dense optical flow in the object tracking.

In this code, our objective is to introduce the principles of optical flow and focus on object tracking using sparse optical flow algorithm – Lucas-Kanade optical flow.

Sparse optical flow

Sparse optical flow is a computer vision technique used to track the motion of distinct, key features within a sequence of images or frames, such as in video analysis. It focuses on tracking a specific set of interest points or features, like corners. By analyzing the displacement of these features between frames, sparse optical flow provides valuable information about object motion and scene dynamics.

One of the main drawbacks of object tracking using sparse optical flow is that it only tracks a set of specific points (i.e., key points) instead of the entire object. While this approach can be computationally efficient and works well for certain scenarios where only specific points of interest need to be tracked, it can also be limiting in cases where we need to track the whole object.

Sparse optical flow is useful for object tracking when:

- we have less computational resources, as it is faster than dense optical flow.

- we need to handle occlusions and changes in illumination in the video.

- we are tracking objects that have distinct features or edges.

Lucas-Kanade method

The Lucas-Kanade optical flow method is a sparse optical flow technique that estimates the motion of specific points or regions in consecutive video frames. It relies on the assumption of constant brightness and calculates local motion vectors by comparing intensity changes around these points, providing information about the speed and direction of motion at those locations.

## Activities

### ACTIVITY 1: TRACKING USING SPARSE OPTICAL FLOW

TASK 1

Manual keypoint selection

1 HOUR

Students´ work:

Your objective is to develop a program which enables user to manually select exactly two keypoints from the image. You should create a window where the user can select keypoints using the mouse and save them in the list. This will be used later as initial points for the optical flow.

Advice:  As keypoints are used in the tracking, when selecting them, select one on top-left part of the object and one on the bottom-right part of the object.mro

Hint: create a callback function for the mouse event 'click' that saves the selected keypoint in the global list. Note clicking event is cv2.EVENT_LBUTTONDOWN.

Teacher’s solution

First, we present how to use OpenCV to select keypoints manually from an image. We create a window where the user can select keypoints using the mouse. The function 'click' is a callback function for the mouse events that saves the selected keypoints in a global list. The code also sets up an infinite loop that displays the image until two keypoints are selected or the 'q' key is pressed.

Selected keypoints are stored in the list 'keypoints' - we will use these in the next code snippet as initial points for the optical flow. Note that converting to grayscale and saving image in variables old_frame and old_gray is done for the next code snippets.

When the frame is shown, please select two keypoints - one on the top-left part of the boat and one on bottom-right part. These two will be used for the tracking process in the next code snippet.

# Create an empty list to store the selected keypoints

keypoints = []

def click(event, x, y, flags, param):

"""

A callback function for mouse event - saving selected keypoints.

"""

global keypoints

if event == cv2.EVENT_LBUTTONDOWN:

keypoints.append([float(x), float(y)])

# Load the image into a numpy array

frame = cv2.imread("./data/got10k/sequence_example/00000001.jpg")

# Convert the color image to grayscale

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

# Create a new window and set the mouse callback function

cv2.namedWindow("Selecting keypoints")

# Set mouse callback to select key points

cv2.setMouseCallback("Selecting keypoints", click)

# As this is the first frame, set the old frame and old gray images

# these two will be used in the next code snippet

old_frame = frame

old_gray = gray

# Show the frame to select the keypoints until 2 keypoints are selected

while True:

cv2.putText(frame, "Click on two keypoints", (10, 100), cv2.FONT_HERSHEY_SIMPLEX, 1, (255, 255, 255), 3)

cv2.imshow("Selecting keypoints", frame)

if cv2.waitKey(1) == ord("q") or len(keypoints) == 2:

break

cv2.destroyAllWindows()

TASK 2

Tracking using sparse optical flow

1 H 30

Students´ work:

For this task, your objective is to develop a program that uses the Lucas-Kanade optical flow algorithm to track an object in a video sequence.

Hint: You can follow next steps:

Loop through all the frames in the directory, except the first one - that is the image that we got inital bounding box points from in the previous snippet.

Convert each frame to grayscale.

Use the points from previous frame to calculate the optical flow between the previous and current frame.

If two points were successfully tracked, draw a bounding box around the object.

Display the frame.

Teacher’s solution

Here, we use the Lucas-Kanade optical flow algorithm to track an object in a video sequence.  The code loops through all the frames in the directory (except the first one, as that is the image that we got inital bounding box points from), reads each frame, and converts it to grayscale.

The code then uses the points from previous frame to calculate the optical flow between the previous and current frame. If two points were successfully tracked, a bounding box is drawn around the object, and the old frame and old gray images are updated. Finally, frame with the bounding box is displayed.

# set the path to the directory containing the frames

frame_dir = "./data/got10k/sequence_example/"

# get a list of all the frame filenames in the directory and sort them

frame_filenames = sorted(os.listdir(frame_dir))

# create Lucas-Kanade tracker object

lk_params = dict(winSize=(15, 15), maxLevel=4, criteria=(cv2.TERM_CRITERIA_EPS | cv2.TERM_CRITERIA_COUNT, 10, 0.03))

# a set of points to track are keypoints selected in previous code snippet

p0 = np.array(keypoints, dtype=np.float32)

p0 = p0.reshape(-1,1,2)

# loop through the frames

for filename in frame_filenames:

# skip the first image

if filename == "00000001.jpg":

continue

# read the frame from file and convert it to grayscale

frame_path = os.path.join(frame_dir, filename)

frame = cv2.imread(frame_path)

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

# calculate optical flow between the old and current frames

p1, st, err = cv2.calcOpticalFlowPyrLK(old_gray, gray, p0, None, **lk_params)

# select the points that were successfully tracked

if p1 is not None:

good_new = p1[st==1]

if(len(good_new) == 2):

# draw the bounding box based on two selected keypoints on the frame

top_left_x, top_left_y = np.array(good_new[0].flatten(), dtype=np.uint32)

bottom_right_x, bottom_right_y = np.array(good_new[1].flatten(), dtype=np.uint32)

frame = cv2.rectangle(frame, (top_left_x, top_left_y), (bottom_right_x, bottom_right_y), color=(255, 0, 0))

# update the old frame and old gray images

old_frame = frame.copy()

old_gray = gray.copy()

p0 = good_new.reshape(-1,1,2)

# display frame

cv2.imshow("Sparse optical flow for object tracking", frame)

if cv2.waitKey(0) == ord("q"):

break

# Close all windows

cv2.destroyAllWindows()